- Install Virtual COM driver under Driver.

- Install PAT2573Loader_V1.0R0_Setup.exe under FWUpdate.

- Run PAT2573Loader in Start Menu -> GIGA-TMS.

- Choose COM for Connection Type.

- Choose COM15.

- Click Connect. Message should show Connected.

- Click APROM to load the FW BIN file

- Click Start to start FW update.

- When PASS is shown next to progress bar, the update is complete.

- Install MSRCfgSetup_V1_4R18_PSW00025.exe, the configuration utility

- Copy files under UtilityPatchFile
MSRCfg.ini to C:\Program Files\MSRCfg\MSR Configure cover
PA-T2573U.cfg to C:\Program Files\MSRCfg\MSR Configure\Default

- Run MSR Configure in Start Menu.

- Click Scan to search COM ports for i-Button reader. The settings page will show up.

- To load configuration file, click Open.

- Various parameters can be altered. However, since this utility was originally written for MSR, then altered for i-Button, some parameter behavior will require trial to find out. The related parameters are under i-Button and Package tabs.

- To update parameter on the reader, click Write.

- Click Test Mode will present a Notepad-like blank area for testing. Click again returns to parameter screen.