FEC XPOS LED SDK

Integration Guide

FEC

Feb. 17th, 2021

Document Revision History

| Rev. | Date | Author | Draft/Changes |
| --- | --- | --- | --- |
| 0.1 | 2020/2/2 | Elliot Ling | First Version |
| 0.2 | 2020/2/17 | Elliot Ling | - Add 32bit dll support - Add sample console test program: “FEC_XPOS_LED_ConsoleTester”. |

- Introduction

  - Overview

This document describes about how to use the FEC XPOS LED SDK to control LED. By importing functions from DLL on windows application

The supported colors are:

- Red

- Green

- Blue

- White

- Cyan-Blue

- Yellow

- Purple

- System Architecture

TBD

The Figure show FEC XPOS LED SDK included software stack of system architecture. FEC XPOS LED SDK support DLL interface for POS application development, Developer can load library with DLL for their POS application.

- System Requirement & Installation

This SDK can be used with the following system configurations

- Hardware Platform (Mother Board)

  - FEC FKLU

  - FH-Z8300/FH-Z8350

- Supported Operating System (OS)

  - Microsoft® Windows Embedded® POSReady 2009

  - Microsoft® Windows Embedded® POSReady 7

  - Microsoft® Windows 10 IOT Enterprise LTSB/LTSC

The following showed installation steps

- Executed FEC_XPOS_LED_SDK_SETUP.exe for starting installation

- Click Install button to start install

- Finish Install for FEC XPOS LED SDK

- FEC XPOS LED SDK will install in C:\Program Files (x86)\FEC\XPOS_LED_SDK。Please also manually set the file attribute to the FEC_XPOS_LED_Utility.exe with the “Executed as administrator”.

And make sure the files: “fec_xpos_led_dll.dll”、 “WinIo64.dll” and “WinIo64.sys” are in the same folder of the FEC_XPOS_LED_Utility.exe (And your own developed application)

- DLL interface for FEC XPOS LED SDK

FEC XPOS LED SDK provide DLL interface to control LED, the DLL name is fec_xpos_led_dll.dll.

API Functions & definitions

- #define COLOR_LED_OFF           0

- #define COLOR_RED               1

- #define COLOR_GREEN             2

- #define COLOR_BLUE              3

- #define COLOR_WHITE             4

- #define COLOR_CYAN_BLUE         5

- #define COLOR_YELLOW            6

- #define COLOR_PURPLE            7

- DLLExport bool Init(void)

This function initializes the IO functions, make sure this function was called before calling function: “bool SetLED(BYTE color)”.

Parameter: None

Return Value:

Fail: false

Success: true

- DLLExport bool DeInit(void)

This function De-initialize the IO functions, make sure this function was called before exit the application.

Parameter: None

Return Value:

Fail: false

Success: true

- DLLExport bool SetLED(BYTE color)

This function

Parameter:

int color:

0: Off the LED

1: Turn on the LED as color Red

2: Turn on the LED as color Green

3: Turn on the LED as color Blue

4: Turn on the LED as color White

5: Turn on the LED as color Cyan-Blue

6: Turn on the LED as color Yellow

7: Turn on the LED as color Purple

Return Value:

Fail: false

Success: true

- Example for DLL interface

FEC_XPOS_LED_Utility: Please refer the sample workspace: “FEC_XPOS_LED_Utility” create by Visual Studio 2015 for the sample code.

The screenshot for FEC_XPOS_LED_Utility:

C#:

[DllImport("fec_xpos_led_dll", CharSet = CharSet.Unicode)]

public static extern bool Init();

[DllImport("fec_xpos_led_dll", CharSet = CharSet.Unicode)]

public static extern bool DeInit();

[DllImport("fec_xpos_led_dll", CharSet = CharSet.Unicode)]

public static extern bool SetLED(int color);

const int COLOR_LED_OFF = 0;

const int COLOR_RED = 1;

const int COLOR_GREEN = 2;

const int COLOR_BLUE = 3;

const int COLOR_WHITE = 4;

const int COLOR_CYAN_BLUE = 5;

const int COLOR_YELLOW = 6;

const int COLOR_PURPLE = 7;

- EXE interface for FEC LED SDK

- Set LED Color
Set the LED color with the parameter 1:

“OFF” or “off”

“RED” or “red”

“GREEN” or “green”

“BLUE” or “blue”

“WHITE” or “white”

“CB” or “cb” for Cyan-Blue

“YELLOW” or “yellow”

“PURPLE” or “purple”

Example:

> FEC_XPOS_LED_ConsoleTester.exe RED

> FEC_XPOS_LED_ConsoleTester.exe off